50  MW  RUSSELL  BIOMASS  PROJECT  -  JUNE  6, 2008                        UTILITY RESTRUCTURING FORUM

A	Project Description and Status

- Description

  - 50 MW design

  - Located in Russell in western MA - 20 miles NW of Springfield

  - Site of former paper company that shut down in 1995

  - Next to Westfield River – plenty of flow

  - Excess wood supply in western MA

  - 5-mile transmission line to WMECO 115 kV system

  - All impacts minor except traffic – 80 trucks per day down the half-mile Main Street

  - 25 Federal, state and local permits required

  - Active opponents – DPU Exemption sought from local zoning

    - Project has made extensive efforts to respond and help Town of Russell

- Development started in Jan 2005

  - Air permit submitted Sept 2005 – Revised for stoker early 2007

  - Final EIR approved by MEPA in March

  - Demanding permit process - expect all approvals by year-end

  - Financing closing expected in first half of 2009

B	Economic/Financial Impact Issues

- Continuing, severe escalation of equipment and construction costs

  - Power plant equipment and construction market demand is heavy

  - Major equipment delivery and construction times stretching out

  - Biomass hard costs over $3,500 per kW

- High diesel fuel prices have increased projected delivered wood prices by 25-30% in past year (60% since 2003)

  - Problem aggravated by natural gas prices (the buyer’s alternative) that remain below fuel oil prices on a dollar per MMBtu basis (gap should narrow in the longer term)

  - Main result:  Biomass plants cannot offer a fixed power sale price (need a cost-pass-through to buyer in order to assure financing)

- Final DEP permit conditions with large cost implications

  - WMA water withdrawal permit – pressure for air cooling

  - NPDES (EPA) water discharge permit – pressure for zero discharge design

  - Wetlands permit conditions a big constraint to transmission line construction access and flexibility

- Utility’s high proposed transmission line annual O&M charge a surprise – will likely vary significantly by utility

- Property taxes

  - Twice as high as 5 years ago strictly because of severe capital cost escalation.  (This is a general biomass industry problem.)

  - 3-4 times as high as gas plants on a $ per MW basis

- Transmission line permitting costs are significant – extensive design work needed in MA

- Federal renewable energy production tax credit extension delayed (uncertain revenue – financing issue)

Consequences of the above are:

- Project financing viability more difficult.  Gas-fired power prices (the buyer’s alternative) will be the key determinant – are currently over 9 cents/kWh.

- Maximum assurance of high REC prices for several years needed – Emerging MA energy bill should help

- Renewable energy development in MA is slowing down – no other MA biomass plant has yet submitted a major permit application, suggesting a minimum 2-3 year delay for the next plant

C	Regulatory and Legislative Issues

- Delay pressure (perhaps inevitable) by environmental entities and opponents on permit processing

  - DEP is attempting to be responsive to renewable energy projects, but protecting permits against possible appeals is a big priority

- Emerging Energy Bill

  - Long-term contracting authority for MA utilities important

  - Increased percent mandate of purchases by MA retail suppliers

  - In the future:  More legislation needed to:

    - Create a shortened DEP permit appeal path through EFSB

    - Ideally need some provision to level the playing field for property taxes

    - Prevent out-of-state utilities from building REC-qualified biomass plants when MA utilities are prohibited